....

(/)

•

~National

D

~....

(/)

Semiconductor

DM54S138/DM74S138,   DM54S139/DM74S139

Decoders/Demultiplexers

General Description

These Schottky-clamped  circuits are designed to be used in high-performance   memory-decoding or data-routing appli• cations,   requiring  very  short  propagation  delay  times.  In high-performance  memory systems these decoders can be used  to  minimize  the  effects  of  system  decoding.  When used with  high-speed  memories,  the  delay times  of these decoders are usually less than the typical access time of the memory.  This means that the effective  system  delay intro• duced by the decoder  is negligible.

The 5138 decodes  one-of-eight  lines, based upon the con• ditions at the three binary select inputs and the three enable inputs.  Two active-low  and  one  active-high  enable  inputs reduce  the  need for  external  gates  or inverters  when  ex• panding. A 24-line decoder can be implemented  with no ex• ternal inverters, and a 32-line decoder requires only one inverter.  Ari enable  input can be used as a data  input  for demultiplexing  applications.

The 5139  comprises  two separate  two-line-to-four-line   de• coders in a single package.  The active-low enable input can be used as a data line in  demultiplexing  applications.

All of these  decoders/demultiplexers   feature  fully buffered inputs,  presenting  only  one  normalized  load to  its  driving circuit.   All   inputs   are   clamped   with   high-performance Schottky diodes to suppress line-ringing and simplify system design.

ConnectionDiagrams

Features

•   Designed specifically  for high  speed: Memory decoders

Data transmission   systems

•   5138  3-to-8-line  decoders  incorporates   3 enable  inputs to simplify cascading  and/or  data reception

•   S139  contains  two  fully  independent  2-to-4-line  decod•

ers/ demultiplexers

•   Schottky  clamped for  high performance

•   Typical propagation  delay time (3 levels of logic)

5138    8 ns

5139    7.5  ns

•   Typical power dissipation

S138    245 mW S139    300 mW

Dual-ln·Une  Package

DATAOUTPUTS

ENABLE

Dual-ln-UnePackage

SELECT                    DATAOUTPUTS

Vee       YO        Y1         Y2        Y3         Y4        Y5        Y6

I 1s         15         14         13        12         11         10         9

Vee       G2         A2         82        2YO       2Y1       2Y2       2Y3

I 16         15          14         13         12          11         10         9

ti                                                        '

I                      D          I}                                   '~

-

I}               I)                                   1)

i                             I                                      •J       I

| 2 | 3 | 4 | 5 | 6 | 7 | je |  | 2 | 3 | 4 | 5 | 6 7 | 1 a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A B c |  | G2A | G2B | G1 | Y7 | GND | ENABLE | A1 | 81 | 1YO | 1Y1 | 1Y2 1Y3 | GND |
|  |  |  |  |  | OUTPUT |  | G1 |  |  |  |  |  |  |

SELECT                        ENABLE                                                                           SELECT                   DATA OUTPUTS

TLIF/6466-1                                                                                                                   TLIF/6466-2

Order Number  DM54S138J, DM54S139J, DM54S138W, DM54139W, DM74S138N or DN74S139N

see NS Package NumberJ16A, N16E or W16A

3-52

Absolute Maximum  Ratings  (Note)

If  Miiitary/Aerospace    specified    devices   are  required,               Note:  The "Absolute  Maximum Ratings"  are those  values please    contact     the    National    Semiconductor    Sales             beyond  which the safety  of the device  cannot  be guaran- Office/Distributors  tor  avaHablllty  and  specHlcatlons.                   teed  The device should not be operated at these limits. The Supply Voltage                                                                      7V            parametric  values defined in the  "Electrics/ Characteristics"

Input  Voltage                                                                       5.5V              table are not guaranteed  at the absolute  maximum ratings.

The "Recommended  Operating Conditions"  table will define

Operating Free Air  Temperature  Range                                               the conditions  for actual device operation.

DM54S                                                    -  ss· c to + 12s·c

DM74S                                                           o· c to+   ro-c

Storage Temperature Range                    -es· Cto  + 150"  c

Recommended OperatingConditions

Symbol                              Parameter

DM54S138,S139

DM74S138,S139

Units

Min            Nom           Max            Min            Nom            Max

Vee                  Supply Voltage                                         4.5                5               5.5            4.75              5              5.25               v V1H                          High Level Input Voltage                           2                                                       2                                                       v V1L                             Low Level Input Voltage                                                                0.8                                                   0.8               v

loH                   High Level Output Current                                                             -1                                                    -1               mA loL                   Low Level Output Current                                                              20                                                    20              mA TA       Free  Air Operating  Temperature            -55                                 125               0                                   70               ·c

ElectricalCharacteristicsover recommended  operating free air temperature (unless otherwise noted)

Symbol                      Parameter                                          Conditions                                 Min

Typ

(Note  1)

Max           Units

v,                  Input  Clamp Voltage                 Vee=Min,11=      -18mA                                                                       -1.2               v

VOH               High  Level  Output                    Vee  =  Min,  loH  =  Max          DM54           2.5                 3.4                                      v

Voltage                                     V1L  =  Max, V1H   =  Min            DM74           2.7                 3.4

Vol                  Low Level Output                     Vee =  Min,  loL =  Max                                                                          0.5               v

Voltage                                     V1H  =  Min,  V1L  =  Max

| 11 Input Current @ Max Vee= Max, V1 = 5.5V Input Voltage |  |  |  |  |  | 1 | mA p.A mA mA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| l1H | High Level Input Current | Vee = Max, v1 = 2.7V |  |  |  | 50 |  |
| l1L | Low Level Input Current | Vee = Max, V1 = 0.5V |  |  |  | -2 |  |
| los | Short Circuit | Vee= Max | DM54 | -40 |  | -100 |  |
|  | Output Current | (Note2) | DM74 | -40 |  | -100 |  |
| Ice | Supply Current (5138) | Vee= Max (Note 3) |  |  | 49 | 74 | mA |
| Ice | Supply Current (Sl 39) | Vee = Max (Note 3) |  |  | 60 | 90 | mA |

Note  1: All typicals  are at Vee  =   5V, TA  =   25'C.

Note  2: Not more than one  output  should  be shorted  at a time,  and the duration  should  not exceed  one second. Note  3: Ice  is measured  with all outputs  enabled  and open.

3-53

'S 138 Switching Characteristics

at Vee =  SV and TA =  25°C (See Section 1  for Test Waveforms and Output Load)

From                                                      RL =  2800

(Input)             Levels

Symbol                    Parameter                         to                of Delay

(Output)

CL=   15pf               CL=   SO pf

Min         Max         Min         Max

Units

1PLH                     Propagation Delay Time              Select to                   2                                     7                                 9                  ns

Low to High Level Output             Output

1PHL                    Propagation Delay Time              Select to                   2                                   10.5                              14               ns

High to Low Level Output              Output

tPLH                Propagation Delay Time              Select to                   3                                    12                               14               ns

Low to High Level Output              {)Jtput

lPHL                Propagation Delay Time              Select to                   3                                    12                               15               ns

High to Low Level Output              Output

tPLH                Propagation Delay Time             Enable to                  2                                     8                                10               ns

Low to High Level Output              Output

tpHL                 Propagation Delay Time             Enable to                  2                                    11                                        14               ns

High to Low Level Output              Output

tpLH                 Propagation Delay Time             Enable to                  3                                    11                               13               ns

Low to High Level Output              Output

tpHL                Propagation Delay Time             Enable to                  3                                    11                               14               ns

High to Low Level Output              Output

'S 139 Switching Characteristics

at Vee  = 5V and TA = 25°C (See Section 1  for Test Waveforms and Output Load)

From                                                      RL =  2800

(Input)             Levels

Symbol                    Parameter                         to                of Delay

(Output)

CL=   15pF              CL=   SO pf

Min         Max         Min         Max

Units

tpLH                 Propagation Delay Time              Select to                   2                                    7.5                               10               ns

Low to High Level Output              Output

tf>HL               Propagation Delay Time              Select to                   2                                    10                               13               ns

High to Low Level Output              Output

tPLH                Propagation Delay Time              Select to                   3                                    12                               13               ns

Low to High Level Output              Output

tpHL                Propagation Delay Time              Select to                   3                                    12                               15               ns

High to Low Level Output              Output

tpLH                 Propagation Delay Time             Enable to                  2                                     8                                10               ns

| Low to High Level Output Output lPHL | Propagation Delay Time | Enable to |
| --- | --- | --- |
|  | High to Low Level Output | Output |

2                                   10                                   13               ns

3-54

Function  Tables

Inputs

Enable     Select

S138                                             S139

Outputs                                                 Inputs

Enable         Select

Outputs

....

=0-

=•

G1 G2" c  B  A   YO Y1 Y2 Y3 Y4  YS Y6 Y7

x	H      x  x   x   H    H    H    H    H     H    H    H L	x    x  x   x   H    H    H    H    H     H    H    H

H     L     L   l    L    L    H    H    H      H    H    H    H H        L     L   L   H     H    L    H    H      H      H     H    H H     L     L   H   L   H    H    L    H    H    H    H    H

G            B        A       YO   Y1    Y2       Y3

H	x      x      H         H           H         H L 	L        L         L          H         H         H L 	 L        H        H         L          H         H L 	H        L         H         H         L          H L 	H       H        H         H         H         L

| H L L H H H H H l H H H H |  |  |  |  |  |  |  |  |  |  |  |  | H = high level, L = low level. X = don't cere (either low or high logic: level) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H | L | H | L | L | H | H | H | H | L | H | H | H |  |
| H | L | H | L | H | H | H | H | H | H | L | H | H |  |
| H | L | H | H | L | H | H | H | H | H | H | L | H |  |
| H | L | H | H | H | H | H | H | H | H | H | H | L |  |

•G2=G2A+G2B

H = high level, L = low level, X = don't care (eitherlow or high logic level)

Logic Diagrams

8138

(15)   YO

(14)

Y1

ENABLE   {:~A(.;.:.;,,) ---a

IHPUTS

(13)  Y2

G2B.-<S..).. _     _.

(12)   Y3

(11)  V4

DATA

OUTPUTS

A  (1)                                                                                                                                  (10)  YS

SELECT

INPUTS

B  (2)

(9)  Y6

c (3)                                                                                                                       (?)   Y7

8139

TL/F/~3

SELECT

l A1 (2)

INPUTS     Bl   (l)

DATA

OUTPUTS

TL/F 16466-4

3-55

This datasheet has been downloaded from:

www.DatasheetCatalog.com

Datasheets for electronic components.